QUESTION 1

Designing and Simulating Register Transfer and ALU Operations

Aim: To design a simple register organization that performs register transfers and basic arithmetic and logic operations using an ALU.

Design:

| Component | Purpose |
| --- | --- |
| R1, R2 | Input/source registers for storing operands |
| R3 | Destination/result register |
| ALU | Performs ADD, SUB, AND and OR operations |
| Control Unit | Selects registers and ALU operation |
| Bus | Transfers data between registers and the ALU |

Register Transfer Operations:

1. R2 ← R1   – transfers the contents of R1 to R2.
2. R3 ← R1 + R2   – ALU adds R1 and R2 and stores the result in R3.
3. R3 ← R1 − R2   – ALU subtracts R2 from R1.
4. R3 ← R1 AND R2   – performs bitwise AND.
5. R3 ← R1 OR R2   – performs bitwise OR.

Example:

Let R1 = 12 and R2 = 5. ADD gives R3 = 17, SUB gives R3 = 7. For binary data, AND and OR are performed bit by bit.

Execution Sequence:

Input → Register selection → Data transfer → ALU operation → Result transfer → Destination register.

Role of ALU:

The ALU is the main processing unit for arithmetic and logical operations. It receives operands from registers, performs the selected operation, and sends the result back to a destination register.

Result:

The designed system successfully demonstrates register transfer, arithmetic operations and logic operations using multiple registers and an ALU.

QUESTION 2

Designing Single-Bus and Multi-Bus Computer Organization

Aim: To model data transfer between registers, memory and I/O using single-bus and multi-bus organization and compare their efficiency.

Design – Single Bus:

A single common bus connects the registers, memory interface, I/O interface and ALU. Since the same bus is shared, only one major transfer can occur at a time.

| Operation | Example |
| --- | --- |
| Register transfer | R2 ← R1 |
| Memory read | R1 ← M[Address] |
| Memory write | M[Address] ← R1 |
| I/O read | R1 ← Input |
| I/O write | Output ← R1 |

Design – Multi-Bus:

A multi-bus organization uses separate buses so that source operands can be transferred to the ALU simultaneously and the result can be transferred to a destination register.

| Feature | Single Bus | Multi Bus |
| --- | --- | --- |
| Data transfer | Mostly sequential | More parallel |
| Hardware | Simple | More complex |
| Bottleneck | Higher | Lower |
| Throughput | Lower | Higher |
| Cost | Lower | Higher |

Working Example:

For R3 ← R1 + R2, a multi-bus design can send R1 and R2 to the ALU through separate paths and write the result to R3. In a single-bus design, the operands normally need separate transfers over the shared bus.

Result:

The multi-bus organization provides better parallel data transfer and reduces bus bottlenecks, while the single-bus organization is simpler and requires less hardware.

QUESTION 3

Designing and Simulating a 5-Stage Instruction Pipeline

Aim: To design a 5-stage instruction pipeline and compare its performance with a non-pipelined processor.

Pipeline Design:

| Stage | Function |
| --- | --- |
| IF – Fetch | Fetch instruction from memory |
| ID – Decode | Decode instruction and read operands |
| EX – Execute | Perform ALU operation |
| MEM – Memory | Read/write memory when required |
| WB – Write-back | Write result to destination register |

Pipeline Execution Example:

| Cycle | I1 | I2 | I3 |
| --- | --- | --- | --- |
| 1 | IF |  |  |
| 2 | ID | IF |  |
| 3 | EX | ID | IF |
| 4 | MEM | EX | ID |
| 5 | WB | MEM | EX |
| 6 |  | WB | MEM |
| 7 |  |  | WB |

Performance Analysis:

In a non-pipelined processor, an instruction completes all stages before the next instruction begins. In a pipelined processor, different instructions occupy different stages simultaneously. For an ideal 5-stage pipeline, the maximum theoretical throughput after filling is one completed instruction per clock cycle.

For k stages and n instructions, ideal pipelined cycles are approximately k + n − 1, while a simple non-pipelined model requires approximately k × n cycles. Therefore, ideal speedup approaches k as the number of instructions becomes large.

Result:

The pipeline increases instruction throughput and reduces the average time per instruction, although hazards and stalls reduce the practical speedup.

QUESTION 4

Designing a Pipeline with Data, Control and Structural Hazards

Aim: To demonstrate the three major pipeline hazards and apply stalling, forwarding and branch prediction to reduce their effect.

Hazard Design:

| Hazard | Example | Effect | Technique |
| --- | --- | --- | --- |
| Data | I2 uses result of I1 | Dependency delay | Forwarding / Stall |
| Control | Branch changes PC | Wrong-path fetch | Branch prediction |
| Structural | Two stages need same resource | Resource conflict | Extra resource / Stall |

Data Hazard Example:

I1: R1 ← R2 + R3
I2: R4 ← R1 + R5
I2 needs the result produced by I1. Data forwarding can send the result from a later pipeline stage directly to the input of the ALU instead of waiting for normal write-back.

Control Hazard Example:

If an instruction is a branch, the processor may not immediately know which instruction should be fetched next. Branch prediction predicts the likely path so useful instructions can continue entering the pipeline.

Structural Hazard Example:

If two instructions require the same hardware resource during the same clock cycle, a conflict occurs. The processor can provide separate resources or insert a stall.

Result:

Hazard handling techniques reduce pipeline stalls and improve instruction throughput. Forwarding is useful for data dependencies, branch prediction helps control hazards, and additional resources or stalls handle structural conflicts.

QUESTION 5

Designing Superscalar, Out-of-Order, Speculative Execution and SMT

Aim: To analyze how modern processors execute multiple instructions in parallel and improve resource utilization.

System Design:

| Technique | Working |
| --- | --- |
| Superscalar | Issues multiple independent instructions per cycle |
| Out-of-order execution | Executes ready instructions before waiting instructions |
| Speculative execution | Executes predicted instruction paths before certainty |
| Hyper-threading | Allows multiple hardware threads to share processor resources |
| SMT | Issues instructions from multiple threads to use execution units efficiently |

Example:

Consider four instructions where I1 is waiting for data, while I2 and I3 are independent. An out-of-order processor can execute I2 and I3 using available execution units instead of leaving those units idle while waiting for I1.

Parallel Execution:

A superscalar processor may issue more than one independent instruction in the same clock cycle. Multiple execution units such as ALUs, load/store units and floating-point units allow different operations to proceed simultaneously.

Performance Comparison:

| Processor approach | Instruction throughput | Resource utilization |
| --- | --- | --- |
| Basic sequential execution | Low | Lower |
| Superscalar | Higher | Higher |
| Out-of-order + speculative | Higher | Better |
| SMT / Hyper-threading | Can increase throughput | Better use of idle resources |

Analysis:

These techniques improve performance by increasing parallelism and keeping processor resources busy. However, actual improvement depends on instruction independence, branch behavior, memory delays and available hardware resources.

Result:

Superscalar execution, out-of-order execution, speculative execution and SMT can significantly improve instruction throughput and resource utilization compared with a basic sequential processor.

ASSESSMENT DETAILS

Course Code: CSA1209 & CSA1215    Course Title: Computer Architecture

CO Assessed: CO3    Assessment: ASSESSMENT TOOL 1 – EXPERIMENTS

Weightage: 40%    Total Marks: 25

CO3 Statement: Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6)

Student Name: ______________________________    Reg.No.: ______________________________

Department: ________________________________    Date: ______________________________

Code of Conduct:

I ____________________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: ______________________________

MARKS ALLOCATION

| Criteria | Understanding of Concepts | Experimental Design | Use of Tools | Analysis of Results | Documentation |
| --- | --- | --- | --- | --- | --- |
| Weightage | 20% | 20% | 25% | 20% | 15% |

Q1     Q2     Q3     Q4     Q5

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |