NAME: PRIYADHARSHINI K S

REG NO: 192511248

COURSE: COMPUTER ARCHITECTURE

CODE: CSA 1204

CHATGPT:

Q1. Compare L1, L2 and L3 Cache Levels

Cache memory is a high-speed memory located close to the processor. Modern processors generally use multiple cache levels: L1, L2 and L3. They differ mainly in capacity, latency, bandwidth and their impact on processor performance.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Closest to CPU core | Near CPU core | Shared among cores |
| Size | Smallest | Medium | Largest |
| Latency | Lowest | Higher than L1 | Higher than L2 |
| Bandwidth | Highest | High | Lower than L1/L2 |
| Access speed | Fastest | Fast | Comparatively slower |
| Purpose | Frequently used data/instructions | Backup for L1 | Shared backup for L1/L2 |

L1 Cache

L1 cache has the lowest access latency and is usually divided into instruction cache and data cache. Since it is very close to the processor core, frequently accessed instructions and data can be retrieved quickly.

L2 Cache

L2 cache is larger than L1 but has slightly higher latency. If required data is not available in L1, the processor checks L2.

L3 Cache

L3 cache is generally larger and may be shared by multiple processor cores. It has higher latency than L1 and L2 but reduces the need to access slower main memory.

Impact on Processor Performance

A high cache hit rate reduces the number of accesses to main memory. This reduces average memory access time and improves processor throughput.

Conclusion:
L1 provides the fastest access, L2 provides a larger secondary cache, and L3 provides a large shared cache. The hierarchy improves processor performance by keeping frequently required data closer to the CPU. The assessment specifically requires comparison of these levels using latency, bandwidth and throughput.

Q2. Compare SRAM and DRAM

SRAM (Static RAM) and DRAM (Dynamic RAM) are two important types of memory used in computer systems.

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full form | Static Random Access Memory | Dynamic Random Access Memory |
| Latency | Very low | Higher than SRAM |
| Speed | Faster | Slower |
| Bandwidth | High | High, but generally lower effective access speed |
| Throughput | High for low-latency access | High for large-capacity transfers |
| Cost | Expensive | Less expensive |
| Density | Lower | Higher |
| Refresh | Not required | Required |
| Main use | Cache memory | Main memory |

SRAM

SRAM stores data using flip-flop circuits and does not require periodic refreshing. Its low latency makes it suitable for processor cache memory.

DRAM

DRAM stores data using capacitors and requires periodic refreshing. It provides higher storage density at lower cost, making it suitable for main memory.

In Cache and Main Memory

SRAM is preferred for cache because the processor needs very fast access to frequently used data.

DRAM is preferred for main memory because computers require larger memory capacity at reasonable cost.

Conclusion

SRAM provides lower latency and is suitable for cache memory, while DRAM provides higher capacity and is suitable for main memory. Therefore, both technologies play different roles in the memory hierarchy.

Q3. Virtual Memory with Paging vs Cache Memory

Virtual memory and cache memory are different mechanisms used to improve computer-system operation.

| Feature | Virtual Memory with Paging | Cache Memory |
| --- | --- | --- |
| Main purpose | Extends available memory using secondary storage | Speeds up CPU memory access |
| Storage | RAM + secondary storage | Small high-speed memory |
| Latency | High when page fault occurs | Very low on cache hit |
| Speed | Slower | Faster |
| Throughput | Lower during page faults | Higher for frequently accessed data |
| Management | Operating system | Hardware/cache controller |
| Unit | Pages | Cache blocks/lines |

Virtual Memory with Paging

Virtual memory allows a program to use an address space larger than the available physical RAM. Memory is divided into fixed-size pages.

When a required page is not present in RAM, a page fault occurs and the operating system loads the page from secondary storage.

This provides greater memory capacity but page faults can significantly increase access latency.

Cache Memory

Cache stores frequently accessed data and instructions close to the processor. When the required data is found in cache, it is accessed much faster than from main memory.

Throughput Comparison

Cache memory improves throughput by reducing the average time required for memory accesses.

Virtual memory improves the ability to run large programs but can reduce system performance when frequent page faults occur.

Conclusion

Cache memory primarily improves access speed and processor throughput, whereas virtual memory with paging primarily provides memory capacity and address-space flexibility. The assessment requires comparison of these mechanisms specifically in terms of access latency and system throughput.

Q4. NAND Flash vs DRAM

NAND Flash and DRAM are different memory technologies with different performance characteristics.

| Feature | NAND Flash | DRAM |
| --- | --- | --- |
| Type | Non-volatile | Volatile |
| Latency | Higher | Lower |
| Speed | Slower for random access | Faster |
| Bandwidth | High for sequential operations | High |
| Refresh | Not required | Required |
| Data retention | Retains data without power | Loses data without power |
| Main use | SSDs, storage | Main memory |
| Hierarchical level | Secondary/storage level | Main-memory level |

NAND Flash

NAND Flash is non-volatile memory. It retains stored information even when power is removed.

It is widely used in SSDs and other storage devices. It has much higher latency than DRAM, especially for random access, but provides persistent storage.

DRAM

DRAM is volatile memory and is normally used as the system's main memory. It provides much faster access compared with NAND Flash.

DRAM is therefore suitable for storing actively running programs and data.

Suitability in Hierarchical Memory

A typical hierarchy can be represented as:

CPU

↓

Cache

↓

DRAM

↓

NAND Flash / SSD

↓

Long-term Storage

DRAM is positioned closer to the processor because of its lower latency, while NAND Flash is used at the storage level because of its non-volatility and high capacity.

Conclusion

DRAM is suitable for high-speed main-memory operations, while NAND Flash is suitable for persistent storage. NAND Flash offers non-volatility, whereas DRAM provides significantly faster access.

Q5. Compare MRAM and RRAM

MRAM (Magnetoresistive RAM) and RRAM (Resistive RAM) are advanced non-volatile memory technologies designed to provide high performance with lower energy consumption.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full form | Magnetoresistive RAM | Resistive RAM |
| Data storage | Magnetic state | Resistance state |
| Volatility | Non-volatile | Non-volatile |
| Access time | Fast | Fast |
| Throughput | High | High |
| Energy efficiency | High | High |
| Data retention | Good | Good |
| Potential use | Embedded memory, cache-like applications | Embedded memory, storage and memory systems |

MRAM

MRAM stores information using magnetic states. Since it is non-volatile, data can remain stored even when power is removed.

MRAM offers fast access and good endurance, making it suitable for applications that require frequent read/write operations.

RRAM

RRAM stores information by changing the resistance of a memory cell between different states. It is also non-volatile and can potentially provide high density and low-power operation.

Performance Comparison

MRAM generally provides fast access and good endurance, while RRAM offers the potential for high density and efficient memory implementation. Both technologies can reduce energy consumption compared with continuously powered volatile memory in suitable applications.

Conclusion

MRAM and RRAM are promising non-volatile memory technologies. Both provide fast access and good energy efficiency, but their underlying storage mechanisms are different. Their suitability depends on requirements such as access time, throughput, density and energy consumption. The assessment specifically asks for comparison of these technologies using access time, throughput and energy efficiency.

GEMINI:

Comparative Analysis: Hierarchical Memory Systems & Advanced Memory Technologies

1. L1, L2, and L3 Cache Levels

| Metric / Parameter | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location / Scope | Integrated within each core | Dedicated per core (or small cluster) | Shared across all CPU cores |
| Typical Capacity | 32 KB – 128 KB per core | 512 KB – 2 MB per core | 8 MB – 64 MB+ (Shared) |
| Latency | 0.5 – 1.5 ns (1–4 clock cycles) | 3 – 7 ns (10–20 clock cycles) | 10 – 20 ns (40–60 clock cycles) |
| Bandwidth | Ultra-high (~1–3 TB/s aggregate) | High (~500–1000 GB/s) | Moderate-High (~200–500 GB/s) |
| Throughput Impact | Maximum sustained pipeline throughput | Mitigates L1 cache miss penalties | Reduces costly off-chip DRAM accesses |

- Processor Performance Impact: The multi-tiered cache hierarchy strikes a balance between execution speed and storage capacity. L1 keeps core execution units fed without stalling execution pipelines. L2 provides a secondary low-latency buffer to capture hit bursts that exceed L1 capacity. L3 acts as a shared synchronization point for multi-core systems, preventing core-to-core bus contention and minimizing high-latency main memory accesses.

2. SRAM vs. DRAM

| Characteristic | SRAM (Static RAM) | DRAM (Dynamic RAM) |
| --- | --- | --- |
| Internal Architecture | 6-transistor (6T) latch circuit | 1-transistor 1-capacitor (1T1C) cell |
| Access Latency | Ultra-fast (~0.5 – 5 ns) | Moderate (~10 – 50 ns) |
| Bandwidth | High (Direct bus line connection) | Lower per pin; optimized via multi-channel architectures |
| Throughput | High throughput without refresh overhead | Moderate throughput; periodic refresh cycles block access |
| Density & Cost | Low cell density, high area footprint, high cost | High cell density, compact footprint, low cost |
| Target Application | On-chip Cache Memory (L1, L2, L3) | Off-chip Main Memory (System RAM) |

- Design Trade-offs: SRAM relies on flip-flop circuits that retain state continuously without requiring refresh cycles, making it ideal for low-latency CPU caches. DRAM stores data as electrical charges in capacitors, requiring dynamic refreshing every few milliseconds. This periodic refresh lowers effective throughput and increases latency, but its high cell density makes DRAM cost-effective for large system capacities.

3. Virtual Memory with Paging vs. Cache Memory

| Characteristic | Cache Memory | Virtual Memory (Paging) |
| --- | --- | --- |
| Managed By | Dedicated Hardware (Cache Controller) | Hardware (MMU / TLB) + Operating System |
| Storage Medium | On-chip SRAM | Off-chip DRAM (RAM) backing up to SSD/HDD |
| Access Latency | ~1 – 20 ns | ~50 ns (DRAM hit) to ~10–100 µs (Page Fault / SSD) |
| Block Size | Cache Line (typically 64 Bytes) | Page (typically 4 KB – 2 MB) |
| Miss Penalty Target | Main Memory Access Latency | Storage Disk I/O Latency |
| Impact on Throughput | Direct instruction pipeline feed | Enables multi-tasking and memory isolation |

- Latency & Throughput Analysis: Cache memory operates transparently to hide the latency gap between CPU cores and main DRAM. Virtual memory uses demand-paging to grant each process a contiguous virtual address space larger than physical memory. Translation Lookaside Buffers (TLB) accelerate page translations; however, a page fault induces disk I/O operations, dropping system throughput significantly compared to hardware cache misses.

4. NAND Flash vs. DRAM

| Metric | DRAM (Dynamic RAM) | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile (Loses data when powered off) | Non-Volatile (Retains data without power) |
| Access Latency | Ultra-low (~10 – 50 ns) | High (Read: ~25–50 µs, Write/Erase: ~100–1000 µs) |
| Bandwidth | Very High (~25 – 100 GB/s per channel) | Moderate (~1 – 7 GB/s via PCIe/NVMe interfaces) |
| Endurance | Infinite write cycles | Limited write cycles (P/E cycles degrade oxide layer) |
| Hierarchical Role | Primary Main Memory (Active execution space) | Secondary / Non-Volatile Storage (SSDs) |

- Hierarchical Memory System Integration: DRAM serves as volatile primary working memory, prioritizing real-time execution bandwidth and low latency. NAND Flash trades lower read/write speeds for non-volatility and dense, low-cost capacity. Modern storage hierarchies use DRAM to stage active data blocks, protecting NAND flash from excessive write wear while buffering system throughput.

5. MRAM vs. RRAM (Emerging Non-Volatile Technologies)

| Parameter | MRAM (Magnetoresistive RAM) | RRAM (Resistive RAM) |
| --- | --- | --- |
| Operating Principle | Magnetic Tunnel Junction (MTJ) resistance variation | Conductive filament formation across dielectric layer |
| Access Time / Latency | Extremely Fast (~1 – 10 ns) | Fast (~10 – 50 ns) |
| Throughput | High (Comparable to L2/L3 SRAM speeds) | High (Slightly lower write bandwidth than MRAM) |
| Energy Efficiency | High (Zero static leakage; ultra-low switching energy) | High (Low operation voltage; favorable for neuromorphic computing) |
| Endurance & Scalability | High endurance (>1012 cycles), moderate density | Moderate endurance (106-108 cycles), 3D-scalable density |

- Performance Analysis: Both MRAM and RRAM combine the speed of SRAM/DRAM with non-volatility. MRAM delivers near-SRAM read/write latencies and high endurance, making it a strong candidate for persistent cache architectures (Unified L3/L4 memory). RRAM excels in high-density 3D integration and low-power analog resistive switching, making it suitable for high-density storage-class memory (SCM) and neuromorphic AI hardware.

PERPLEXITY:

#Comparative Analysis of Hierarchical Memory Systems

1. L1, L2 and L3 Cache

Cache memory is organized into levels so that the processor can obtain frequently used data quickly while still providing adequate storage capacity. All cache levels normally use SRAM, but they differ in size, location, sharing, latency and bandwidth.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Typical location | Inside each processor core | Usually private to a core | Usually shared by multiple cores |
| Typical capacity | 32–64 KB per core | 256 KB–1 MB per core | 8–64 MB or more |
| Approximate latency | 1–2 ns; about 3–5 cycles | 3–6 ns; about 10–20 cycles | 10–25 ns; about 40–50 cycles |
| Bandwidth | Highest | Very high, lower than L1 | High, lower than L2 |
| Throughput | Highest for small, repeated accesses | High for moderate working sets | High for shared data and larger working sets |
| Main purpose | Immediate instruction and data access | Reduce accesses to L3 and DRAM | Reduce expensive DRAM accesses |

Representative teaching examples place L1 below 5 cycles, L2 below 20 cycles, L3 below 50 cycles and DRAM above 100 cycles. Other measured or architecture-dependent estimates give approximately 2 ns for L1, 6 ns for L2, 15–25 ns for L3 and about 100 ns for DRAM. These values vary between processor generations. [ics.uci](https://ics.uci.edu/~swjun/courses/2021S-CS250B/material/lec2%20-%20Cache%20And%20Memory%20System.pdf)

### Impact on processor performance

- **L1 hits** allow the CPU to continue with very little delay. L1 is small because larger caches require longer lookup paths and consume more chip area.

- **L2 cache** provides a balance between capacity and speed. It catches data that does not fit in L1 and prevents many requests from reaching the shared cache.

- **L3 cache** is slower but much larger. In multicore systems, it can allow cores to share data without repeatedly accessing external DRAM.

- A **cache miss** causes the processor to search the next level. Therefore, a miss in L1 but a hit in L2 is relatively inexpensive, while a miss in all cache levels leads to a much slower DRAM access.

- Cache throughput depends not only on raw bandwidth but also on hit rate, associativity, cache-line size, prefetching and contention between cores.

The average memory access time can be represented as:

\[

AMAT = T_{L1} + MR_{L1}(T_{L2} + MR_{L2}(T_{L3} + MR_{L3}T_{DRAM}))

\]

where \(MR\) is the miss rate at a cache level. Thus, improving the L1 hit rate can have a greater performance effect than merely increasing the capacity of L3.

2. SRAM and DRAM

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Storage cell | Flip-flop, commonly six transistors | Capacitor and transistor |
| Refresh requirement | Not required while powered | Required periodically |
| Latency | Approximately 0.5–2.5 ns for fast on-chip SRAM | Approximately 50–100 ns |
| Bandwidth | Very high for on-chip operation | High aggregate bandwidth, but greater access delay |
| Throughput | Excellent for small random accesses | Excellent for large sequential or burst transfers |
| Density | Low | Much higher |
| Cost per bit | High | Low |
| Typical use | Registers and CPU caches | Main memory |

SRAM is significantly faster because its stored bit remains available in a stable circuit and does not require refresh before use. DRAM stores charge in a capacitor; the charge leaks and must be refreshed. Representative figures identify SRAM latency around 0.5–2.5 ns and DRAM latency around 50–70 ns. [eng.auburn](https://www.eng.auburn.edu/~nelsovp/courses/elec5200_6200/ELEC5200_6200%20Memory.pdf)

### Cache and main-memory design

SRAM is appropriate for L1, L2 and L3 caches because cache capacity is relatively small and speed is more important than cost or density. Its disadvantages are large cell size, high transistor count and greater area per bit.

DRAM is used for main memory because it provides much greater capacity at lower cost. Its row activation, sensing, precharging and refresh operations increase latency. However, DRAM transfers data in bursts, so its sustained bandwidth can be high when the processor accesses consecutive cache lines.

For example, a processor may experience a short delay when reading a cache line from SRAM, whereas a DRAM access may take tens of nanoseconds before the first data arrives. Once a DRAM row is open, subsequent burst transfers can be considerably more efficient. Therefore:

- SRAM gives better **random-access latency**.

- DRAM gives better **capacity and cost efficiency**.

- DRAM can provide high **sustained throughput** for sequential traffic.

- SRAM generally provides higher **per-access throughput** and lower contention inside the processor.

3. Virtual Memory, Paging and Cache Memory

Cache memory and virtual memory both exploit locality, but they operate at different levels and serve different purposes.

| Aspect | Cache memory | Virtual memory with paging |
| --- | --- | --- |
| Main purpose | Reduce average CPU memory-access time | Give processes a large logical address space and protection |
| Managed by | Hardware cache controller | Operating system, page tables and MMU |
| Transfer unit | Cache line, commonly 64 bytes | Page, commonly several KB |
| Normal access | SRAM cache lookup | TLB and page-table translation followed by DRAM access |
| Miss penalty | Next cache level or DRAM | Page fault and storage access |
| Latency | Very low on a hit | Higher because of translation and possible OS activity |
| Effect on throughput | Usually improves CPU throughput | Increases capacity but can reduce throughput during faults |

A cache miss normally causes a lookup in another cache level or DRAM. A virtual-memory translation first uses the TLB. On a TLB miss, the processor may perform a page-table walk; if the page is absent from physical memory, a page fault transfers it from storage. [cs.cornell](https://www.cs.cornell.edu/courses/cs4410/2019su/lectures/11-vm-caching.pdf)

The effective access time for demand paging can be expressed as:

\[

EAT=(1-p) \times \text{memory access time}

+p \times \text{page-fault service time}

\]

where \(p\) is the page-fault probability. The page-fault service time includes operating-system overhead and, when necessary, storage read/write operations. [bbau.ac](https://www.bbau.ac.in/dept/dit/TM/Virtual%20Memory%20and%20Demand%20Paging.pdf)

### Throughput analysis

Cache memory generally improves throughput because it reduces the number of long-latency DRAM accesses. It is especially effective when a program has temporal locality, such as repeatedly using the same array elements.

Paging improves system-level throughput by allowing several programs to coexist even when their combined virtual address spaces exceed physical RAM. However, excessive page faults cause **thrashing**. In that situation, the processor spends more time moving pages between DRAM and storage than executing instructions.

The distinction is therefore important:

- Cache memory improves **speed** by keeping small, frequently used blocks close to the CPU.

- Paging improves **capacity, isolation and multiprogramming**.

- A cache miss is usually a hardware event with a relatively modest penalty.

- A major page fault may involve storage and can be thousands or millions of times slower than an SRAM or DRAM access.

A good system uses both mechanisms: virtual addresses are translated through the TLB and page tables, while the resulting physical addresses are used to search the cache hierarchy.

4. NAND Flash and DRAM

| Feature | NAND Flash | DRAM |
| --- | --- | --- |
| Volatility | Non-volatile | Volatile |
| Typical first-access latency | Tens of microseconds | Tens of nanoseconds |
| Read operation | Page-oriented | Random access with burst transfer |
| Write operation | Much slower; erase-before-program constraints | Faster and more flexible |
| Bandwidth | Lower, especially for random accesses | Much higher for active-row and burst accesses |
| Endurance | Limited program/erase cycles | No comparable write-cycle limitation |
| Density | Very high | Moderate |
| Typical role | SSDs, persistent storage, lower memory tiers | Main memory |

NAND Flash usually reads an entire page rather than a single byte. Example lecture data gives about 25 µs to obtain the first byte from NAND Flash, followed by approximately 40 MiB/s for subsequent data, while SDRAM may provide the first byte in about 40 ns and subsequent data at approximately 4.8 GB/s. A 2 KiB transfer was therefore shown as roughly 75 µs from Flash compared with about 500 ns from SDRAM. [passlab.github](https://passlab.github.io/CSCE513/notes/lecture10_MemTechLocality.pdf)

### Suitability for hierarchical memory

DRAM is suitable for the main-memory level because it offers low enough latency for active programs and high bandwidth for CPU and multicore traffic. NAND Flash is unsuitable as a direct replacement for DRAM in the normal CPU load/store path because its page-level access, slow writes and erase operations create large delays.

NAND Flash is valuable in lower levels of the hierarchy because it provides:

- Non-volatility.

- Very high storage density.

- Lower cost per bit.

- Lower standby power than continuously refreshed DRAM.

- Persistent storage for operating systems, applications and data.

NAND may also be used in emerging storage-class-memory designs, but controllers, buffering, wear levelling, error correction and parallel channels are needed to hide its limitations.

5. MRAM and RRAM

MRAM and RRAM are non-volatile memory technologies intended to combine persistence with faster access than conventional Flash. Their exact performance depends strongly on the cell type, process technology, array size and memory controller.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Storage principle | Magnetic state, commonly magnetic tunnel junctions | Resistance change caused by formation or rupture of a conductive filament |
| Read access | Commonly a few to tens of nanoseconds | Potentially sub-nanosecond to several nanoseconds |
| Write access | Fast, commonly a few to tens of nanoseconds | Potentially fast, but often variable |
| Throughput | High and predictable for embedded memory | Potentially very high due to small cells and fast switching |
| Energy | Very low standby power; write energy varies by type | Very low read/write energy is possible |
| Endurance | Excellent, often 1010to 1015cycles depending on design | Potentially above 1012cycles, but variability and reliability remain concerns |
| Main strengths | Speed, endurance, mature embedded integration | Density, low energy and scalability |
| Main limitations | Write current, cell area and scaling challenges | Resistance variability, filament control and device uniformity |

A recent review reports MRAM read speeds below 5 ns for SOT-MRAM, 5–20 ns for STT-MRAM and 20–50 ns for Toggle-MRAM. The same comparison lists DRAM at approximately 10–30 ns and 3D NAND Flash at approximately 50–100 µs.  Another review describes MRAM read/write times in the 1–10 ns range and reports that RRAM can achieve sub-nanosecond operation and sub-picojoule energy in suitable implementations. [pmc.ncbi.nlm.nih](https://pmc.ncbi.nlm.nih.gov/articles/PMC13363608/)

### Performance and energy analysis

MRAM provides fast, byte-addressable non-volatile storage with excellent endurance. It is therefore suitable for embedded caches, microcontroller memory, processor scratchpads and applications requiring instant-on operation. Its major challenge is that writing the magnetic state may require more energy or current than reading it.

RRAM uses a resistance state rather than charge or magnetization. Its simple cell structure can support high density and low switching energy. It is attractive for embedded non-volatile memory and in-memory computing, where the physical resistance can participate in computation. However, RRAM performance may vary between cells, and forming voltage, resistance drift, write variability and endurance must be controlled.

The most appropriate choice depends on the objective:

- Select **MRAM** when predictable access time, high endurance and low standby power are priorities.

- Select **RRAM** when density, low switching energy and neuromorphic or in-memory-computing capability are priorities.

- Neither technology universally replaces SRAM, DRAM or NAND Flash; each fills a different point in the latency–density–energy trade-off.

## Conclusion

The memory hierarchy works because no single technology optimizes latency, bandwidth, capacity, cost, persistence and energy at the same time. L1, L2 and L3 caches use fast SRAM to reduce processor stalls; DRAM supplies dense, high-capacity main memory; virtual memory and paging extend the logical address space; NAND Flash provides persistent high-density storage; and MRAM/RRAM offer promising non-volatile alternatives between conventional SRAM, DRAM and Flash. The best architecture combines these technologies according to workload locality, multicore contention, capacity requirements and energy constraints.